# Space, Weight, and Power Impacts of Neuromorphic Approaches for Small Unmanned Systems

Gregory Tozzi

June 15, 2026

Literature Review – First Full Draft

# Abstract

# Introduction

# Literature Review/Related Work

We present work related to our objective of quantifying space, weight, and power (SWaP) impacts of neuromorphic approaches to on-device hyperspectral image classification and trajectory prediction for small unmanned systems.

## Applications of Neuromorphic Computing

Neuromorphic computing is an emerging field of research that focuses on applying brain-inspired hardware and software solutions to learning tasks [1]. While the bulk of the work in artificial neural networks (ANNs) centers on networks constructed of nodes and edges to which floating point values are applied, the main effort in neuromorphic approaches finds inspiration from binary spiking mechanisms more closely related to the current understanding of how biological neurons and synapses function [1].

Neuromorphic algorithms have been applied to computer vision and sequence-to-sequence tasks. In the field of computer vision, convolutional neural networks (CNNs) designed for standard CPU-GPU compute have been successfully converted to spiking neural networks (SNNs), the predominant neuromorphic architecture without significant loss of accuracy [2]. The success of such ANN-to-SNN conversions supports the conclusion that the computational capacity of SNNs is on par with that of ANNs [3]. Exploiting the temporal nature of SNNs directly, approaches to applying neuromorphic architecture to sequence-to-sequence tasks like language translation and sentiment analysis have shown competitive performance relative to recurrent neural networks (RNNs) with the advantages of reduced power consumption and parameter efficiency [4].

The principal practical reason to pursue neuromorphic solutions is that neuromorphic techniques, like spiking neural networks, offer power and memory advantages over ANNs. Every connection in an ANN is activated during every forward pass through the network [1]. In contrast, neurons in an SNN transmit discrete spikes once their activation threshold is achieved, resulting in dramatically less computation activity [1]. The relative reduction in activity between SNNs and ANNs performing comparable tasks increases with network depth, allowing SNNs to scale more efficiently [2]. The computations that do occur in the SNN are dramatically simpler than in an equivalent ANN insofar as ANNs require multiply-and-accumulate operations because weights in ANNs are floating point values . In contrast, SNNs transmit binary spikes and only require accumulate operations [1, 4]. The sparsity of SNNs also yields reductions in parameter counts relative to recurrent architectures performing similar tasks, reducing memory overhead [4]. Reducing power and memory requirements is especially consequential for small edge systems where payloads and power are severely constrained.

## Neuromorphic Computing Applied to Small Unmanned Systems

Small unmanned systems are particularly SWaP constrained [5], and neuromorphic techniques have been applied to elements of their operation. Conventional deep neural networks running on GPUs frequently require hundreds of watts of power, making them poorly suited for deployment on small unmanned platforms operating under strict size, weight, and power budgets [6]. The event-driven, sparse computation of SNNs is well-matched to the low-power, low-latency sensing and processing requirements of small edge systems [6].

Neuromorphic techniques have been applied to remote sensing and spectral image classification, demonstrating the feasibility of such approaches to support image classification in small autonomous systems. Hyperspectral image classification is a computationally intensive task that has been difficult to deploy on small edge systems using conventional ANNs [7]. Applying SNNs to hyperspectral classification has been accomplished with high accuracy and an approximately ten times reduction in energy consumption compared with CNNs performing the same classification tasks [7]. Lossless conversion of ANNs to SNNs for remote sensing scene classification tasks has been demonstrated using aerial image datasets [8]. These efforts demonstrate the feasibility of achieving some of the promised gains associated with neuromorphic computing to small unmanned systems.

Neuromorphic techniques are well-suited to event-driven motion estimation due to the temporal processing strength inherent in spiking neural networks; in fact, the power and latency benefits of spiking algorithms are best realized in applications that take specific advantage of the temporal nature of SNNs [9]. Optical flow estimation from event cameras via an SNN, a representative application of value to unmanned systems, has been shown to operate at ten times the frequency with 58% of the energy consumption of a baseline deep learning approach [10]. Vision-based drone navigation, with its inherent spatiotemporal nature, is also well-suited to treatment with spiking networks [6].

Because rigorous SWaP benchmarking is methodologically difficult, an outstanding research question is the quantification of the specific benefit in terms of space, weight, and power (SWaP) savings to small unmanned systems offered by adopting neuromorphic techniques.

# Research objectives

# Experimental Design/Methodology

# Analysis plan

# References

[1]	K. Roy, A. Jaiswal, and P. Panda, “Towards spike-based machine intelligence with neuromorphic computing,” Nature, vol. 575, no. 7784, pp. 607–617, Nov. 2019, doi: 10.1038/s41586-019-1677-2.

[2]	A. Sengupta, Y. Ye, R. Wang, C. Liu, and K. Roy, “Going Deeper in Spiking Neural Networks: VGG and Residual Architectures,” Front. Neurosci., vol. 13, p. 95, Mar. 2019, doi: 10.3389/fnins.2019.00095.

[3]	N. Rathi et al., “Exploring Neuromorphic Computing Based on Spiking Neural Networks: Algorithms to Hardware,” ACM Comput. Surv., vol. 55, no. 12, pp. 1–49, Dec. 2023, doi: 10.1145/3571155.

[4]	N. Rathi and K. Roy, “LITE-SNN: Leveraging Inherent Dynamics to Train Energy-Efficient Spiking Neural Networks for Sequential Learning,” IEEE Trans. Cogn. Dev. Syst., vol. 16, no. 6, pp. 1905–1914, Dec. 2024, doi: 10.1109/TCDS.2024.3396431.

[5]	K. H. Jones and J. N. Gross, “Reducing Size, Weight, and Power (SWaP) of Perception Systems in Small Autonomous Aerial Systems,” in 14th AIAA Aviation Technology, Integration, and Operations Conference, American Institute of Aeronautics and Astronautics. doi: 10.2514/6.2014-2705.

[6]	S. S. Chowdhury, D. Sharma, A. Kosta, and K. Roy, “Neuromorphic computing for robotic vision: algorithms to hardware advances,” Commun Eng, vol. 4, no. 1, p. 152, Aug. 2025, doi: 10.1038/s44172-025-00492-5.

[7]	Y. Liu, K. Cao, R. Li, H. Zhang, and L. Zhou, “Hyperspectral Image Classification of Brain-Inspired Spiking Neural Network Based on Approximate Derivative Algorithm,” IEEE Trans. Geosci. Remote Sensing, vol. 60, pp. 1–16, 2022, doi: 10.1109/TGRS.2022.3207098.

[8]	S. Wu, J. Li, L. Qi, Z. Liu, and X. Gao, “Remote Sensing Imagery Scene Classification Based on Spiking Neural Network,” in 2021 IEEE International Geoscience and Remote Sensing Symposium IGARSS, Brussels, Belgium: IEEE, Jul. 2021, pp. 2795–2798. doi: 10.1109/IGARSS47720.2021.9554883.

[9]	M. Davies et al., “Advancing Neuromorphic Computing With Loihi: A Survey of Results and Outlook,” Proc. IEEE, vol. 109, no. 5, pp. 911–934, May 2021, doi: 10.1109/JPROC.2021.3067593.

[10]	W. Ponghiran, C. M. Liyanagedera, and K. Roy, “Event-based Temporally Dense Optical Flow Estimation with Sequential Learning,” in 2023 IEEE/CVF International Conference on Computer Vision (ICCV), Paris, France: IEEE, Oct. 2023, pp. 9793–9802. doi: 10.1109/ICCV51070.2023.00901.